## Fiji workflow for binary conversion and manual microglial review:

## See video for detailed instruction.

- Define the region of interest (ROI) using the Polygon tool. Clear outside of the polygon tool and save the ROI-defined image.

- Open the original image, (not the ROI-defined version), create a Composite Image, and save this file.

  - This will allow you to toggle on/off different channels efficiently when you begin doing your quality control using the ManualMicroglialComparison.ijm

- Open and run EditedBinaryConversion.ijm using the predetermined thresholding method.

  - Use the Ciernia Lab’s Step 1 if you wish to test Auto threshold methods.

  - Test additional global thresholding options by editing the setThreshold step.

- Save the output image. This is your thresholded image.

  - Name it: originalimagename.tif_thresholded.tif

- Open and run ManualMicroglialComparison.ijm, then work through the macro as follows:

  - Open the binary image when prompted.

    - Indicate whether pre-saved ROIs are available:

      - If yes, load the ROI ZIP file (this is only done if you previously started analyzing this video).

      - If no, the macro will automatically detect ROIs from the binary image (this is usually what you will want to do).

  - Open the corresponding composite image when prompted (this is the image that you created in step 2).

    - Check the box indicating that this is an RGB image, so Fiji opens the Channels Tool. This allows individual channels to be toggled on and off to check for nuclei/microglial cell staining.

  - The macro will then move through each ROI one at a time and zoom both the thresholded image and the original image to the same location.

    - At each ROI:

      - Inspect the region in both the original image and the thresholded image. Use the Channels Tool to toggle channels on an off and inspect for nuclei.

      - Use Cmd+ or Cmd- to zoom in and zoom out.

      - Make any manual edits needed, such as filling unwanted regions with the Fill tool or separating connected microglial cell with the Pencil tool (set to 2px).

    - Press OK or click Enter on your keyboard to continue through each ROI.

    - Optional actions during review include:

      - Entering an ROI number to jump to a specific ROI. Otherwise, the macro will continue sequentially.

      - Checking Mark Microglia to flag that ROI and allow you to add an optional note.

      - Type ‘f’ to fill the selected ROI black in the binary image. This performs the same action as using the Fill tool manually, but more efficiently. Note, it fills the entire selected ROI regardless of if you added divisions, so in the case when you have made manual edits or are viewing an ROI that is not the selected ROI you should manually use the Fill tool.

  - After all ROIs have been reviewed, the macro will display a summary of the marked ROIs. This is an option feature, if you wish to return to a couple and reinspect or send any to someone else to get a second opinion.

  - Save the final edited binary image.

- Continue with Step 3 (single cell extraction) and Step 4 (skeletonization) from the Ciernia Lab.

Note: This process should likely be completed for all images before beginning the Ciernia lab pipeline for single-cell extraction and skeletonization. However, it is recommended to first run through the full workflow on one thresholded image to ensure that you are comfortable with the steps and that everything functions properly in your dataset and analysis pipeline.

Note: The manual microglia editor was created to help make the process efficient. However, if you find aspects of your workflow that could be improved the script can easily be modified/functionality can be added.